CS 303 LAB6 Project EGG TIMER

Down Counter

To create a down-counter we need to create multiple if statements in Verilog which checks the current number and decrement it accordingly when the count is "1".

At the beginning we make sure that zero output becomes "1" only when every position in the clock is "0".

First, we check if the load input is asserted and if it is "1" we assign the numbers to our digital clock that was set by the pins on the board.

Secondly if the “rst” button is pressed we need to set our clock to "0 0 0 0" which how it was at the beginning.

After that if the controller tells us that we should start counting (count = 1):

We check if the right most digit is 0 and if it is not, we decrement it by one.

When the Right Most Digit becomes "0" we look for next digit (left of it) and decrement it by 1 and set the RMD to max value (9)

We continue like this by checking if they are 0 or not. when we find a zero digit, we set the digits which are on the right of it to their maximum values. (9-9.5-9)

We continue checking until every digit is 0 when all of them are “0”, zero output becomes 1 and the down counter is stopped by the controller.

Controller

To set the "Controller" first we check if the "Start” button is pressed or not.

if pressed "start" output becomes "1"; (that means the counter can start counting 999 to 0)

if not "start" output becomes "0".

Second, we check if the "Preset" button is pressed or not.

if pressed "load" output becomes "1"; (that means down-counter can load the inputs to the circuit)

if not "load" output becomes "0".

Third, we check if the "endd" is "0" or "1"

if "endd" is "0" that means the counter did not finish its job then the circuit needs to wait until

"endd" becomes "1".

Lastly, we check the "zero" condition which occurs when all the outputs from the down-counter become "0"

if "zero" is "1" then that means we need to set the circuit as it was at the beginning and stop counting.

Emir Asal 27933, Utku Alkan 28227